Tech Byte – Live Streaming Basics

Time: 1 hour

Purpose:
By the end of this activity, participants will understand the fundamentals of live streaming — including equipment setup, audio and lighting configuration, and key considerations for creating a smooth, professional-looking broadcast.

What You’ll Need:

- Video camera & tripod

- Photix lighting kit

- RØDE audio kit

- Laptop per group with OBS Studio installed

- Chair for presenter

- Power cables and adapters

- Reliable high-speed internet connection

Steps

1. Choose the Right Location

- Pick a quiet, well-ventilated area with minimal background noise.

- Ensure access to power outlets for all equipment.

- Clear clutter or set up a backdrop for a tidy appearance.

- Arrange any props or materials for the shoot.

2. Position the Video Camera

- Mount the camera securely on a tripod, at the presenter’s eye level.

- Frame the shot so the presenter is centered with enough headroom.

3. Set Up Studio Lights

- Position two lights at 45° angles from the presenter for even illumination.

- Adjust brightness and color temperature to suit the environment.

4. Connect the RØDE Audio Kit

- Plug the RØDE receiver into the laptop (USB-C).

- Connect one transmitter to the presenter (use both for interviews).

- Optionally use the handheld microphone.

- Power on transmitters/receivers and confirm audio levels are detected.

5. Configure OBS Studio

- Open OBS Studio and create a new Scene.

- Add video input (camera via USB or HDMI capture card).

- Add audio input (RØDE audio kit).

- Match OBS settings to the camera (e.g., 1080p, 30fps).

6. Run a Lighting & Audio Test

- Check for shadows or overexposure and adjust lights.

- Test microphone sync and audio clarity in OBS.

7. Lights, Camera, Action!

- Record a short 1-minute interview using OBS Studio.

- Discuss what makes a good studio setup.

- Explore common streaming platforms: YouTube Live, Facebook Live, and Twitch

  - This session provides an introduction to live streaming — there won’t be time to cover streaming platforms in depth. If participants are interested, consider planning a follow-up workshop or inviting the SLQ Digital Inclusion Team to deliver a full session on setting up live streams and managing online broadcasts.

Staff Notes:

- Promote the session through the Digital and You app.

- Guide participants through each equipment step.

- Once the setup is complete and tested, assign recording roles:

  - Camera Operator

  - OBS Studio Manager (Laptop)

  - 1–2 Presenters

- Encourage discussion about potential community uses for live streaming (e.g., festivals, weddings, sports events, local storytelling).

- Offer the library’s live-streaming equipment for loan where appropriate.