| Hammer Drill |  |
| --- | --- |
| DO NOT use this machine unless you have completed the induction and a supervisor has given you permission |  |
| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained or restrained. |
| Appropriate protective footwear with uppers must be worn. | Hearing protection will be required when operating this machinery. |
| Rings and jewellery must not be worn. | A mask must be worn when excessive airborne dust is created. |

### PRE-OPERATIONAL SAFETY CHECKS

- Ensure this power tool has a suitable safe work area.

- Examine the power lead and plug for obvious damage.

- Do not use dull or damaged drill bits.

- Check the selected drill bit is correctly fitted.

- Faulty equipment must not be used. Immediately report suspect equipment.

#### OPERATIONAL SAFETY CHECKS

- Do not connect to power source until all adjustments have been made.

- Check that the power lead does not create a trip hazard and that it is well clear of the workpiece.

- Examine the material to be drilled for splits, loose knots & nails, etc.

- Select the correct sized drill bit.  Tighten securely in the chuck.

- Ensure the workpiece is secure & well supported in a convenient position for drilling.

- The power drill must be held firmly with both hands to control operational accuracy and the rotational torque.

- Keep hands and fingers well clear of moving parts. Avoid blocking & covering the motor ventilation slots with your hands.

- Allow the drill to reach operating speed, then apply load gradually.  Do not apply excessive force.

- Back the drill bit out to clear away all waste.

- Avoid prolonged use as this could overheat the motor.

- Turn off after backing out the drill bit.  Do not place the drill down until the bit has stopped rotating.

#### HOUSEKEEPING

- On completion, Remove drill bit and leave the equipment and work area in a safe, clean and tidy state.

## POTENTIAL HAZARDS

Moving, rotating & sharp parts   Contact with bit    Dusts    Eye injury    Noise                      Burns (friction)