| Hand Router |  |
| --- | --- |
| DO NOT use this machine unless you have completed the induction and a supervisor has given you permission |  |
| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained or restrained. |
| Appropriate protective footwear with uppers must be worn. | Hearing protection will be required when operating this machinery. |
| Rings and jewellery must not be worn. | A mask must be worn when excessive airborne dust is created. |

### PRE-OPERATIONAL SAFETY CHECKS

- Ensure this power tool has a suitable safe work area.

- Before using the router, examine the power cord, extension lead, plugs, sockets and power outlet for damage.

- Check the router bit carefully for cracks or damage before installing it into the router spindle.

- Secure and support the workpiece using clamps or bench vices.

- Ensure the cord does not create a tripping hazard.

- Faulty equipment must not be used. Immediately report suspect equipment.

#### OPERATIONAL SAFETY CHECKS

- Check the workpiece for faults and defects.

- Fit the correct cutter bit to the machine. Ensure cutter bit conforms to machine specifications.

- Make all adjustments for depth of cut BEFORE connecting to the power source.

- Ensure your workpiece is firmly secure & supported.

- Keep fingers, hands and power cord clear of the bit.

- Always consider the direction of rotation. This will determine the direction the router is to travel.

- Allow the router to reach operating speed, then apply load gradually.  Maintain a constant pace to avoid uneven finishes.

- Keep the sole plate pressed firmly on the workpiece.

- Maintain complete control. Always operate with both hands. Maintain a proper & steady footing at all times.

- Make a series of shallow cuts when creating a deep recess.  This is much safer and easier on the tool bit.

- Do not apply excessive force – this could cause the cutter bit to burn the workpiece.

- Turn off immediately after use. Do not place the router down until the cutter has stopped rotating.

#### HOUSEKEEPING

- On completion, Leave the equipment and work area in a safe, clean and tidy state.

## POTENTIAL HAZARDS

Moving, rotating & sharp parts   Contact with cutting blade    Dusts    Kickback                                    Movement of the workpiece    Ejected material    Eye injury      Noise