| Jig Saw Jig Saw |  |
| --- | --- |
| DO NOT use this machine unless you have completed the induction and a supervisor has given you permission |  |
| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained or restrained. |
| Appropriate protective footwear with uppers must be worn. | Hearing protection will be required when operating this machinery. |
| Rings and jewellery must not be worn. | A mask must be worn when excessive airborne dust is created. |

### PRE-OPERATIONAL SAFETY CHECKS

- Ensure this power tool has a suitable safe work area.

- Make sure blade is intact & suits the material to be cut.

- Inspect the workpiece to ensure that there aren't any items which might damage the cutting blade or cause injury to the operator.

- Secure and support the workpiece using clamps or bench vices.

- Plan a sawing procedure to minimise backing out.

- Faulty equipment must not be used. Immediately report suspect equipment.

#### OPERATIONAL SAFETY CHECKS

- Examine the material to be cut for splits, loose knots & nails, etc.

- Ensure the workpiece is secure & well supported.

- Check the underside of the cut to ensure that the blade will not cause any damage or be obstructed.

- Allow the machine to reach full speed before starting a cut – always saw at a moderate rate.

- Keep hands away from in front of the cutting operation.

- Do not apply excessive force to the material being cut.

- Take care when cutting curves to ensure that the blade does not twist or bind - if necessary drill small holes.

- If the blade binds, release the switch immediately – free the blade & inspect for damage.

- Do not allow waste to accumulate on or around the saw blade or the workpiece.  Ensure that off-cuts will not cause binding, or fall on your feet.

- On completion, wait for the blade to stop moving before lifting the jig saw from the workpiece, and then rest the saw on its side, protecting the blade.

- If the blade breaks, stop sawing immediately. Switch the machine off & report it.

- Bring the machine to a complete standstill and disconnect the battery before changing the blade or making any adjustments.

#### HOUSEKEEPING

- On completion, Leave the equipment and work area in a safe, clean and tidy state.

## POTENTIAL HAZARDS

Moving, rotating & sharp parts   Contact with blade    Dusts    Movement of the workpiece     Eye injury      Noise        Manual handling