DRILL
(Cordless Hand drill)

## Scope

This document is intended to estimate potential human health and environmental risks posed by current and potential future conditions at State Library of Queensland (State Library) Fabrication Lab Facility. The risk assessment describes the approach to the risk assessment and facilitates appropriate ways to evaluate current and future risks.

Refer to the Safe Operating Procedures (SOP) for information regarding the safe usage and check list for this equipment.

| Plant/Equipment Description: Drill (18 volt cordless) |  |
| --- | --- |
| Leaders: Daniel Flood |  |
| Locations: Fabrication Lab |  |
| Assessment Date: 02/03/2020 | Review Date: 02/03/2021 |

N.B. This assessment can remain active for up to 5 years. However, an annual monitoring and review process should be undertaken and recorded – refer to the last page of this document.

Below are the details of the manufacturing or production processes attributed to this item of equipment categorised by their assessed inherent risk levels (refer to the Equipment/Process Risk Matrix). The actions required for approval for each level of inherent risk are mandatory.

| Inherent Risk Level |  | Details of Processes | Action Required/Approval |
| --- | --- | --- | --- |
|  | Low | - When drilling smaller holes in various materials, particularly wood produces, using a standard twist drill. - When the work piece is securely held in vice or similar to reduce the possibility of unexpected movement, slipping or drill bit breakage. | - Manage through regular planning processes |
|  | Medium | - When drilling into harder or highly resistant materials. - When the manufacturing process requires a pilot hole to be drilled prior to producing the larger, desired result. - When drilling at high speeds, or when drilling at increased depths. - When using the portable hand drill with larger hole saw bits, spade bits and auger bits, etc. - When using the Hammer Drill or Impact Drill features of this power tool. | - Document controls in planning documents and/or complete this Plant Risk Assessment. |

Minimum standards

## Minimum qualifications and experience Listed below are the general “minimum” recommendations for the management of this Plant/Equipment. Indicate the minimum management controls.

X State Library staff with experience, ability and competency in the safe use of this plant/equipment (indicate one or more of the following): X Specific knowledge of the safe and correct use of this plant/equipment X Experience (i.e. previous involvement and familiarity) in the safe use of this plant/equipment X Demonstrated expertise, ability and competency with this plant/equipment Documented qualifications relating to the use of this plant/equipment (e.g. in a staff profile) OR X A Contractor, other than a State Library staff member, with: X Expertise in the safe and correct use of this plant/equipment Documented qualifications that demonstrate experience, ability and competency in the safe use of this plant/equipment.

X Will any staff require initial and/or ongoing training for the safe use of this plant/equipment? If yes, give details: Initial safety training of new staff

X Will members be operating this plant/equipment? If yes, state how member use of this plant/equipment will be managed (e.g. Workshop Safety Induction) Give details: Safety Induction and under general supervision

Further information if required:

## Minimum control requirements

Supporting documentation available in the school on this plant/equipment includes: X Operators Manual X Safe Operating Procedures (SOP) X Equipment Maintenance Records (EMR) X A process for recording members safety induction e.g. Member induction register X A process for recording staff training and experience, e.g. Staff induction register

X All guards are in place and in good working order for this plant/equipment

Safe Working Zones are defined for this plant/equipment (e.g. yellow lines and/or appropriate signage)

X Suitable personal protective equipment (PPE) is available to be used by all operators

X This plant/equipment complies with relevant safety standards

Further information if required:

Hazards and control measures

Listed below are indicative hazards/risks and suggested control measures. These are by no means exhaustive lists. Add details of any other hazards/risks or additional controls you intend to implement.

Indicate the control measures adopted. Detail their implementation and any additional controls required.

| ## Hazards/Risks | Hierarchy of Recommended Control Measures | ## Yes | ## No | ## Details of how this will be implemented ## (and any additional controls) |
| --- | --- | --- | --- | --- |
| Exposure to Rotating or Moving Parts: - Entanglement and Entrapment Could hair, clothing, ties, jewellery or other materials become entangled with moving parts of plant or materials in motion? - Striking Could anyone be struck by moving objects such as the work piece being ejected, or by the unexpected or uncontrolled movement of the plant or work piece? - Crushing and Pinching Could anyone be crushed or pinched due to falling, uncontrolled or unexpected movement of plant or its load tipping or rolling over, or contact with moving parts during testing, inspection or maintenance? - Cutting, Stabbing and Puncturing Can anyone be cut, stabbed or punctured by coming into contact with moving plant or parts, or objects such as ejected work pieces or waste? | - Where possible, potentially hazardous portable power tools, including all power drills (240v), are substituted or replaced with less hazardous alternatives. | X |  | Supervisor to consider the requirements and alternates |
|  | - All necessary guards and safety devices are in place protecting workers from all moving parts including the rotating chuck and drill bit. | X |  | As per the manufacturer’s standards |
|  | - Staff and member training is provided to minimise exposure to these hazards and risks. | X |  | Safety induction |
|  | - Safe operating procedures (SOPs) are available and clearly displayed. | X |  | Located with equipment and in SOP folder |
|  | - Warning “Danger” tags (or similar) are affixed to all portable power drills under repair or maintenance preventing workers from using the equipment. | X |  | Standard LOTO Procedures |
|  | - “Safe Working Zones” are clearly defined in all workspaces where portable power drills are to be used. |  | X | Supervisor to assess work zone requirement |
|  | - Operators are required to remove all jewellery, tuck in loose clothing and tie back long hair. | X |  | As per SOP requirements |
|  | - All approved personal protective equipment (PPE) is used where required. | X |  | All PPE is provided as per SOP requirements |
| Slips, Trips, Falls and Abrasions: Can anyone using the plant or in the vicinity of the plant, slip, trip or fall due to the working environment or other factors? e.g. Poor housekeeping, dust on floors, slippery or uneven work surfaces, power cables across work areas causing injuries and abrasions? | - Slip resistant flooring is encouraged in workspaces. Regular checks are made for unsafe wear and damage. Inspections are made for any power leads, etc. | X |  | Anti-slip mats available if required |
|  | - Procedures are in place for the disposal of all waste materials around all workspaces where electric hand drilling activities are to be performed. | X |  | Storage & waste disposal procedures |
|  | - Staff training is provided to minimise exposure to these hazards. | X |  | Safety induction |
| Environmental: - Noise Is it likely that the normal operation of this plant will produce excessive noise levels? - Dust, Fumes and Vapours Is it likely there will be airborne dust particles, toxic fumes or volatile vapours produced and therefore be present in the workspace? - Vibration Is the normal operation of this plant likely to create severe or excess vibration that could be transferable to the operator? - Lighting Is there insufficient lighting to operate this plant in a safe manner? Is there a possible strobe lighting effect caused by faulty fluorescent tubes in the workspace? | - Portable power drills are regularly inspected and maintained to help minimise the risk of exposures to these hazards. | X |  | Routine checks and maintenance |
|  | - All portable power tool maintenance is documented. | X |  | Service records |
|  | - Exposure to noisy workshop environments is monitored and evaluated regularly for all workers. | X |  | Monitoring of excess noise during operations by supervisor |
|  | - Staff and member training is provided to minimise exposure to these hazards. | X |  | Safety induction |
|  | - Good lighting is provided to all workspaces and this is maintained on a regular basis. Fluorescent tubes are checked and replaced as required. | X |  | As per workspace risk assessment and housekeeping procedures |
|  | - All approved personal protective equipment (PPE) is used where required. | X |  | All PPE is provided. As per SOP requirements |
| Electrical: Can the operator be injured by electrical shock due to working near or contacting with damaged or poorly maintained live electrical conductors such as power outlets, extension leads, safety switches, starters and isolators or casual water on the floor near plant and machinery? | - Visually checks are made of all portable cordless power tools, their electrical switches, plugs, leads and battery charges. | X |  | Routine checks and maintenance |
|  | - Electrical safety inspections, testing and tagging etc. are completed regularly as per guidelines for all cordless portable power tool battery chargers. | X |  | Annually. As per QLD WHS requirements |
|  | - Warning “Danger” tags (or similar) are affixed to all portable cordless power tools or their battery chargers when under repair or maintenance preventing workers from using them. | X |  | LOTO procedures |
|  | - Electrical maintenance on all portable cordless power tools and chargers is documented. | X |  | Electrical register |
| Exposure: - Friction Is the plant likely to generate heat by friction? Could the plant operator accidentally come into contact with moving materials or machinery components resulting in friction burns to the skin, particularly hands? - Hazardous Substances Is it likely that the plant operator or others nearby in the workspace could be exposed to hazardous or toxic chemicals such as oils, hydraulic fluids, greases, coolants, volatile vapours, fumes or airborne dust particulates? | - Portable power drills are regularly inspected and maintained to help minimise the risk of exposures to these hazards. | X |  | Routine maintenance and servicing |
|  | - All portable power tools maintenance is documented. | X |  | Service records |
|  | - Any potentially hazardous waste materials or toxic dusts and vapours resulting from this drilling process are monitored and managed. | X |  | Dust extraction and air filter monitoring by Supervisor |
|  | - Staff and student training is provided to minimise exposure to these hazards. | X |  | General induction training and housekeeping procedures |
|  | - “Safe Working Zones” are clearly defined in all workspaces where power drills are to be used. | X |  | Supervisor to assess workspace requirements |
|  | - All approved personal protective equipment (PPE) is used where required. | X |  | All PPE is provided. As per SOP requirements |
| Ergonomics and Manual Handling: Can the plant be safely operated, in a suitable location, providing clear and unobstructed access? Poorly designed work stations often necessitate teachers and students performing manual tasks involving heavy lifting and lowering, pushing, pulling or carrying, etc. Such tasks then contribute to a range of musculoskeletal sprains and strains for workers. | - Where possible, practical work benches are planned and adjusted to a comfortable work height thus minimising any unsafe or excessively strenuous manual tasks. | X |  | Use of standard working bench heights and adjustable stands as required. As per workspace risk assessment. |
|  | - Sufficient workspace is provided in all practical classrooms to help ensure unobstructed, safe operation. | X |  | Supervisor to assess workspace requirements |
|  | - Floors are regularly cleaned and free of excessive wood dust, waste materials and other extraneous objects. | X |  | As per general housekeeping peocedures |
|  | - Staff training is provided with regard to manual handling techniques and procedures to minimise exposure to these hazards. | X |  | Staff safety and manual handling inductions. |
| Explosion and Fire: As a consequence of using this particular item of plant and equipment, could anyone be injured by the release of stored energy triggered by volatile, explosive substances such as stored gasses, vapours or liquids? | - Fire extinguishers of the correct type are readily available in all workspaces and positioned near exit doorways. | X |  | As per Australian Standards |
|  | - Staff training is provided regarding procedures for the correct and appropriate use of fire safety equipment. | X |  | Annual safety training |
|  | - Exits from buildings and other work areas are defined and access to them kept clear of obstructions. | X |  | As per Australian building code |
|  | - Safety signage is posted clearly denoting the location of all fire safety items and emergency exits. | X |  | As per Australian building code |

| ## Other Hazards/Risks | ## Additional Control Measures These would relate to the specific student needs, locations and conditions in which you are conducting your activity. |
| --- | --- |

| ## Approval |  |  |  |
| --- | --- | --- | --- |
| ## Submitted by: Simon McKellar |  |  | ## Date: 02/03/2020 |
|  | Approved as submitted. |  |  |
|  | Approved with the following condition(s): |  |  |
|  | Not Approved for the following reason(s): |  |  |
| By: |  | Designation: |  |
| Signed: |  | Date: |  |

| ## Staff members involved in the use of this risk assessment and the associated plant and equipment: |  |
| --- | --- |
|  | Signature: ……………………………….. Date: Signature: ……………………………….. Date: Signature: ……………………………….. Date: Signature: ……………………………….. Date: Signature: ……………………………….. Date: Signature: ……………………………….. Date: Signature: ……………………………….. Date: Signature: ……………………………….. Date: |

## Monitoring and Review This Plant and Equipment Risk Assessment is to be monitored and reviewed annually for a further four (4) years.

| Review 1: |  | Yes | No |
| --- | --- | --- | --- |
| - Are allocated risk levels and “Actions required” unchanged over the past 12 months? - Are Minimum Standards and Recommended Control Measures unchanged over 12 months? - Staffing details have remained unchanged over the past 12 months? |  |  |  |
| If the responses are “NO” for any question, record current details here, and list all staff changes (with signatures) |  |  |  |
| Reviewed by: | Designation: |  |  |
| Signed: | Review Date : |  |  |

| Review 2: |  | Yes | No |
| --- | --- | --- | --- |
| - Are allocated risk levels and “Actions required” unchanged over the past 12 months? - Are Minimum Standards and Recommended Control Measures unchanged over 12 months? - Staffing details have remained unchanged over the past 12 months? |  |  |  |
| If the responses are “NO” for any question, record current details here, and list all staff changes (with signatures) |  |  |  |
| Reviewed by: | Designation: |  |  |
| Signed: | Review Date : |  |  |

| Review 3: |  | Yes | No |
| --- | --- | --- | --- |
| - Are allocated risk levels and “Actions required” unchanged over the past 12 months? - Are Minimum Standards and Recommended Control Measures unchanged over 12 months? - Staffing details have remained unchanged over the past 12 months? |  |  |  |
| If the responses are “NO” for any question, record current details here, and list all staff changes (with signatures) |  |  |  |
| Reviewed by: | Designation: |  |  |
| Signed: | Review Date : |  |  |

| Review 4: |  | Yes | No |
| --- | --- | --- | --- |
| - Are allocated risk levels and “Actions required” unchanged over the past 12 months? - Are Minimum Standards and Recommended Control Measures unchanged over 12 months? - Staffing details have remained unchanged over the past 12 months? |  |  |  |
| If the responses are “NO” for any question, record current details here, and list all staff changes (with signatures) |  |  |  |
| Reviewed by: | Designation: |  |  |
| Signed: | Review Date : |  |  |

© State of Queensland (State Library of Queensland) 2020

This policy is licensed under a Creative Commons Attribution 3.0 Australia licence. You are free to copy, communicate and adapt this work, so long as you attribute the State Library of Queensland.

For more information see http://creativecommons.org/licenses/by/3.0/au