JIGSAW

Scope

This document is intended to estimate potential human health and environmental risks posed by current and potential future conditions at State Library of Queensland (Sate Library) Fabrication Lab Facility. The risk assessment describes the approach to the risk assessment and facilitates appropriate ways to evaluate current and future risks.

Refer to the Safe Operating Procedures (SOP) for information regarding the safe usage and checklist for this equipment.

| Plant/Equipment Description: Jigsaw (various brands) (corded or cordless) |  |
| --- | --- |
| Leader: Daniel Flood |  |
| Locations: Fabrication lab |  |
| Assessment Date: 02/03/2020 | Review Date: 02/03/2021 |

N.B. This assessment can remain active for up to 5 years. However, an annual monitoring and review process should be undertaken and recorded – refer to the last page of this document.

Below are the details of the manufacturing or production processes attributed to this item of equipment categorised by their assessed inherent risk levels (refer to the Equipment/Process Risk Matrix). The actions required for approval for each level of inherent risk are mandatory.

| Inherent Risk Level |  | Details of Processes | Action Required/Approval |
| --- | --- | --- | --- |
|  | Medium | - When cutting straight lines on uniformly square wood materials or plastic sheet between 3mm – 10mm thickness. - When materials are securely clamped down or held in a vice. - When the power cord is securely out of the path of the reciprocating blade. - When members are closely supervised at all times.. | - Document controls in planning documents and/or complete this Plant Risk Assessment. |
|  | High | - When cutting tight curves or angles on materials where the blade could twist or bind. - When cutting any sheet metal materials. - When larger size work pieces or materials cannot be clamped down or stabilised securely.. | - Lead Supervisor approval prior to conducting this activity is required. - Completing a JSA is recommended. |

Minimum standards

## Minimum qualifications and experience Listed below are the general “minimum” recommendations for the management of this Plant/Equipment. Indicate the minimum management controls.

X State Library Staff with experience, ability and competency in the safe use of this plant/equipment (indicate one or more of the following): X Specific knowledge of the safe and correct use of this plant/equipment X Experience (i.e. previous involvement and familiarity) in the safe use of this plant/equipment X Demonstrated expertise, ability and competency with this plant/equipment Documented qualifications relating to the use of this plant/equipment (e.g. in a staff profile) OR X A Contractor, other than a State Library staff member, with: X Expertise in the safe and correct use of this plant/equipment X Documented qualifications that demonstrate experience, ability and competency in the safe use of this plant/equipment.

Will any staff require initial and/or ongoing training for the safe use of this plant/equipment? If yes, give details:

Will members be operating this plant/equipment? If yes, state how student use of this plant/equipment will be managed (e.g. Workshop Safety Induction) Give details: Safety induction required and under direct supervision

Further information if required:

## Minimum control requirements

Supporting documentation available in the school on this plant/equipment includes: X Operators Manual X Safe Operating Procedures (SOP) X Equipment Maintenance Records (EMR) X A process for recording student safety induction e.g. member induction register X A process for recording staff training and experience, e.g. Staff induction register

X All guards are in place and in good working order for this plant/equipment

Safe Working Zones are defined for this plant/equipment (e.g. yellow lines and/or appropriate signage)

X Suitable personal protective equipment (PPE) is available to be used by all operators

X This plant/equipment complies with relevant safety standards

Further information if required: A safe work zone is to be defined by the supervisor for each usage situation.

Hazards and control measures

Listed below are indicative hazards/risks and suggested control measures. These are by no means exhaustive lists. Add details of any other hazards/risks or additional controls you intend to implement.

Indicate the control measures adopted. Detail their implementation and any additional controls required.

| ## Hazards/Risks | Hierarchy of Recommended Control Measures | ## Yes | ## No | ## Details of how this will be implemented ## (and any additional controls) |
| --- | --- | --- | --- | --- |
| Exposure to Rotating or Moving Parts: - Entanglement and Entrapment Could hair, clothing, ties, jewellery or other materials become entangled with moving parts of plant or materials in motion? - Striking Could anyone be struck by moving objects such as the work piece being ejected, or by the unexpected or uncontrolled movement of the plant or work piece? - Cutting, Stabbing and Puncturing Can anyone be cut, stabbed or punctured by coming into contact with moving plant or parts, or objects such as ejected work piece or waste? | - Where possible, potentially hazardous portable power tools, including all jigsaws, are substituted or replaced with less hazardous alternatives. | X |  | Supervisor to consider the requirements and alternatives |
|  | - All necessary guards and safety devices are in place protecting workers from all moving parts including the reciprocating blade. | X |  | AS per the manufacturer’s standards |
|  | - Staff and members training is provided to minimise exposure to these hazards and risks. | X |  | General safety induction and housekeeping procedures |
|  | - Safe operating procedures (SOP’s) are available and clearly displayed. | X |  | Stored with equipment and in SOP folder |
|  | - Warning “Danger” tags (or similar) are affixed to all portable power tools under repair or maintenance preventing workers from using the equipment. | X |  | Standard LOTO procedures |
|  | - “Safe Working Zones” are clearly defined in workspaces where jigsaws are to be used. |  | X | Supervisor to assess and address requirements for safe working zone. |
|  | - Operators are required to remove all jewellery, tuck in loose clothing and tie back long hair. | X |  | As per SOP requirements |
|  | - All approved personal protective equipment (PPE) is used where required | X |  | All PPE is provided as per SOP requirements |
| Slips, Trips, Falls and Abrasions: Can anyone using the plant or in the vicinity of the plant, slip, trip or fall due to the working environment or other factors? e.g. Poor housekeeping, dust on floors, slippery or uneven work surfaces, power cables across work areas causing injuries and abrasions? | - Slip resistant flooring is encouraged in workspaces. Regular checks are made for unsafe wear and damage. Inspections are made for any power leads, etc. | X |  | Anti-slip mats available if required |
|  | - Procedures are in place for the disposal of all waste materials around all workspaces where portable jig sawing activities are to be performed. | X |  | Storage & waste disposal procedures |
|  | - Staff training is provided to minimise exposure to these hazards. | X |  | Safety induction |
| Environmental: - Noise Is it likely that the normal operation of this plant will produce excessive noise levels? - Dust, Fumes and Vapours Is it likely there will be airborne dust particles, toxic fumes or volatile vapours produced and therefore be present in the workspace? - Vibration Is the normal operation of this plant likely to create severe or excess vibration that could be transferable to the operator? - Lighting Is there insufficient lighting to operate this plant in a safe manner? Is there a possible strobe lighting effect caused by faulty fluorescent tubes in the workspace? | - Portable jigsaw tools are regularly maintained to help minimise the risk of exposures to these hazards. | X |  | Routine checks and maintenance |
|  | - All portable power tool maintenance is documented. | X |  | Service records |
|  | - Exposure to noisy workshop environments is monitored and evaluated regularly for all workers. | X |  | Monitoring of excess noise during operations by supervisor |
|  | - Staff and member training is provided to minimise exposure to these hazards. | X |  | General induction training and housekeeping procedures |
|  | - All ducted dust and fume extraction systems are connected and operational, fully maintained and cleaned as required. | X |  | Use of portable dust extraction, workspace air filter and/or dust masks |
|  | - Good lighting is provided to all workspaces and this is maintained on a regular basis. Fluorescent tubes are checked and replaced as required. | X |  | As per workspace risk assessment and housekeeping procedures |
|  | - All approved personal protective equipment (PPE) is used where required. | X |  | All PPE is provided as per SOP requirements |
| Electrical: Can the operator be injured by electrical shock due to working near or contacting with damaged or poorly maintained live electrical conductors such as power outlets, extension leads, safety switches, starters and isolators or casual water on the floor near plant and machinery? | - Visual checks are made of all portable power tools, their electrical switches, plugs and power leads, etc. | X |  | Routine checks and maintenance |
|  | - Electrical safety inspections, testing and tagging, etc. are completed regularly as per guidelines for all portable power tools. | X |  | Annual test and tag for corded and battery charging units |
|  | - Warning “Danger” tags (or similar) are affixed to all portable power tools under repair or maintenance preventing workers from using them. | X |  | Standard LOTO procedures |
|  | - Electrical maintenance on all portable power tools is documented. | X |  | Service records |
| Exposure: - Friction Is the plant likely to generate heat by friction? Could the plant operator accidentally come into contact with moving materials or machinery components resulting in friction burns to the skin, particularly hands? - Hazardous Substances Is it likely that the plant operator or others nearby in the workspace could be exposed to hazardous or toxic chemicals such as airborne toxic wood dust particulates? | - Portable jigsaw tools are regularly maintained to help minimise the risk of exposures to these hazards | X |  | Routine maintenance and servicing |
|  | - All portable power tool maintenance is documented. | X |  | Service records |
|  | - Any potentially hazardous waste materials or toxic dusts resulting from this jig sawing process are monitored and managed. | X |  | Use of dust extraction and P2/P3 face masks if required |
|  | - Staff and member training is provided to minimise exposure to these hazards. | X |  | General Induction and Housekeeping procedures |
|  | - “Safe Working Zones” are clearly defined in all workspaces where jigsaws are used. |  | X | Defined by Supervisor or JSA |
|  | - All approved personal protective equipment (PPE) is used where required. | X |  | All PPE is provided as per SOP requirements |
| Ergonomics and Manual Handling: Can the plant be safely operated, in a suitable location, providing clear and unobstructed access? Poorly designed work stations often necessitate teachers and students performing manual tasks involving heavy lifting and lowering, pushing, pulling or carrying, etc. Such tasks then contribute to a range of musculoskeletal sprains and strains for workers. | - Where possible, practical work benches are planned and adjusted to a comfortable work height, thus minimizing any unsafe or excessively strenuous manual tasks. | X |  | Use of standard working bench heights and adjustable stands as required |
|  | - Sufficient workspace is provided to help ensure unobstructed, safe operation. | X |  | Supervisor to assess work space requirements |
|  | - Floors are regularly cleaned and free of excessive wood dust, waste materials and other extraneous objects. | X |  | As per standard housekeeping procedures |
|  | - Staff training is provided with regard to manual handling techniques and procedures to minimise exposure to these hazards. | X |  | Staff safety and manual handling training. |
| Explosion and Fire: As a consequence of using this particular item of plant and equipment, could anyone be injured by the release of stored energy triggered by volatile, explosive substances such as stored gasses, vapours or liquids? | - Fire extinguishers of the correct type are readily available in all workspaces and positioned near exit doorways | X |  | As per Australian Standards |
|  | - Staff training is provided regarding procedures for the correct and appropriate use of fire safety equipment. | X |  | Staff Fire & Evac training. |
|  | - Exits from buildings and other work areas are defined and access to them kept clear of obstructions. | X |  | As per Australian building codes |
|  | - Safety signage is posted clearly denoting the location of all fire safety items and emergency exits. | X |  | As per Australian building codes |

| ## Other Hazards/Risks | ## Additional Control Measures These would relate to the specific student needs, locations and conditions in which you are conducting your activity. |
| --- | --- |

| ## Approval |  |  |  |
| --- | --- | --- | --- |
| ## Submitted by: Simon McKellar |  |  | ## Date: 02/03/2020 |
|  | Approved as submitted. |  |  |
|  | Approved with the following condition(s): |  |  |
|  | Not Approved for the following reason(s): |  |  |
| By: |  | Designation: |  |
| Signed: |  | Date: |  |

| ## Staff members involved in the use of this risk assessment and the associated plant and equipment: |  |
| --- | --- |
|  | Signature: ……………………………….. Date: Signature: ……………………………….. Date: Signature: ……………………………….. Date: Signature: ……………………………….. Date: Signature: ……………………………….. Date: Signature: ……………………………….. Date: Signature: ……………………………….. Date: Signature: ……………………………….. Date: |

## Monitoring and Review This Plant and Equipment Risk Assessment is to be monitored and reviewed annually for a further four (4) years.

| Review 1: |  | Yes | No |
| --- | --- | --- | --- |
| - Are allocated risk levels and “Actions required” unchanged over the past 12 months? - Are Minimum Standards and Recommended Control Measures unchanged over 12 months? - Staffing details have remained unchanged over the past 12 months? |  |  |  |
| If the responses are “NO” for any question, record current details here, and list all staff changes (with signatures) |  |  |  |
| Reviewed by: | Designation: |  |  |
| Signed: | Review Date : |  |  |

| Review 2: |  | Yes | No |
| --- | --- | --- | --- |
| - Are allocated risk levels and “Actions required” unchanged over the past 12 months? - Are Minimum Standards and Recommended Control Measures unchanged over 12 months? - Staffing details have remained unchanged over the past 12 months? |  |  |  |
| If the responses are “NO” for any question, record current details here, and list all staff changes (with signatures) |  |  |  |
| Reviewed by: | Designation: |  |  |
| Signed: | Review Date : |  |  |

| Review 3: |  | Yes | No |
| --- | --- | --- | --- |
| - Are allocated risk levels and “Actions required” unchanged over the past 12 months? - Are Minimum Standards and Recommended Control Measures unchanged over 12 months? - Staffing details have remained unchanged over the past 12 months? |  |  |  |
| If the responses are “NO” for any question, record current details here, and list all staff changes (with signatures) |  |  |  |
| Reviewed by: | Designation: |  |  |
| Signed: | Review Date : |  |  |

| Review 4: |  | Yes | No |
| --- | --- | --- | --- |
| - Are allocated risk levels and “Actions required” unchanged over the past 12 months? - Are Minimum Standards and Recommended Control Measures unchanged over 12 months? - Staffing details have remained unchanged over the past 12 months? |  |  |  |
| If the responses are “NO” for any question, record current details here, and list all staff changes (with signatures) |  |  |  |
| Reviewed by: | Designation: |  |  |
| Signed: | Review Date : |  |  |

© State of Queensland (State Library of Queensland) 2020

This policy is licensed under a Creative Commons Attribution 3.0 Australia licence. You are free to copy, communicate and adapt this work, so long as you attribute the State Library of Queensland.

For more information see http://creativecommons.org/licenses/by/3.0/au