| SAFE OPERATING PROCEDURE DRILL PRESS |  |
| --- | --- |
| DO NOT use this machine unless you have completed the induction and a supervisor has given you permission |  |
| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained or constrained. |
| Appropriate footwear with substantial uppers must be worn. | Close fitting, protective clothing or a workshop apron is encouraged. |
| Rings and jewellery must not be worn. | Hearing protection may be required for some drilling operations. |

### PRE-OPERATIONAL SAFETY CHECKS

- Check workspace and walkways to ensure no slip-hazards are present.

- Check that the drill chuck guard is in position.

- Ensure the chuck key (if used) has been removed from the drill chuck.

- Locate and ensure you are familiar with the operation of the ON/OFF starter and E-Stop.

- Follow correct clamping procedures to ensure work is secure.

- If the job obstructs the walkway erect a barricade.

- Adjust spindle speed to suit drill, cutter diameter and type of material.

- Faulty equipment must not be used. Immediately report suspect equipment.

##### OPERATIONAL SAFETY CHECKS

- Never leave the Drill Press while it is running.

- Before making adjustments or before cleaning accumulated swarf, switch off and bring the machine to a complete standstill.

- Feed downwards at a sufficient rate to keep the drill cutting.

- Feed with care as the drill breaks through the underside of the work.

- Use a safe working posture.  Beware of clothing or hair catching in the drill chuck or bit.

##### HOUSEKEEPING

- Switch off the machine.

- Leave the machine in a safe, clean and tidy state.

##### POTENTIAL HAZARDS

Entanglement in rotating spindle or drill       Eye injuries       Flying swarf or chips

Sharp edges & burrs       Hot surfaces or drill bits

This SOP does not necessarily cover all possible hazards associated with the tool and should be used in conjunction with other references. It is designed to be used as an adjunct to teaching Safety Procedures and to act as a reminder to users prior to tool use.

© State of Queensland (State Library of Queensland) 2020

This policy is licensed under a Creative Commons Attribution 3.0 Australia licence. You are free to copy, communicate and adapt this work, so long as you attribute the State Library of Queensland.

For more information see http://creativecommons.org/licenses/by/3.0/au